# Bode Plots

Let’s discuss how to do a Bode Plot

Ex: Ts=s-1*s-1000s-10*s-100= s2-1001s+1000s2-110s+1000

T(s) approaches 1 as                  0

T(s) approaches 1 as                  ∞

Using MatLab

N=[1, -1001, 1000]

N =           1       -1001        1000

>> D=[1, -110, 1000]

D =           1        -110        1000

>> freqs(N,D)

- So the magnitude plot starts at 1 at  = 0 at a slope of 0

- It runs horizontally until  = 1 where the slope becomes +6 dB per octave (+20 dB per decade) since it passes a 0

- It continues at this slope until the next break point (the pole at  = 10) where the slope drops back to zero.

- The next break point is the pole at  = 100 so the slope becomes -6 dB per octave

- Then at the last break point (the zero at  = 1000) the slope returns to zero.

This method works great as long as the magnitude is defined at either  = 0 or 
 = ∞ (run the process backwards).  If neither limit is well defined (±∞),

- find a spot along the plot where the slope runs constant and determine the magnitude at the center of that run.

- Now work outward from that point in both directions to draw the piecewise linear approximation to the Bode magnitude plot.